🚦 IoT URL-Based Highway Toll Traffic Light System Complete Technical Documentation ESP32 Core Controller; ENC28J60 Ethernet Module; Arduino Nano Relay Driver.

# 1.  Project Overview

This system is a smart, web-controlled traffic light solution specifically engineered for highway toll booths and controlled access checkpoints. Using a combination of an ESP32 microcontroller, ENC28J60 Ethernet module, and Arduino Nano with relay-driven LED arrays, operators can switch between STOP (Red) and GO (Green) signals via simple URL-based HTTP commands over a local network.

| 🌐 Web-Controlled Change light state via URL commands from any browser on the network. | ⚡ High-Power LEDs 354 LEDs per unit split between red and green high-power arrays. | 🔁 Relay Switching 8-channel relay module provides robust isolation between logic and power circuits. |
| --- | --- | --- |

# 2.  Working Principle

## 2.1  System Architecture

The system operates in two distinct layers: a network/control layer and a power/switching layer.

| Layer 1 — Network / Control - ESP32 hosts a lightweight HTTP web server - ENC28J60 provides Ethernet connectivity via SPI - Operator sends GET request to a URL endpoint - ESP32 parses route and sets GPIO output pins | Layer 2 — Power / Switching - Arduino Nano monitors signal lines from ESP32 - Nano drives 8-channel relay module (LN1–LN8) - Relays switch high-power LED driver circuits - Red array = STOP signal; Green array = GO |
| --- | --- |

## 2.2  Signal Flow

Browser/Operator → HTTP GET Request → ENC28J60 (Ethernet) → ESP32 → GPIO Pins → Arduino Nano → 8CH Relay → LED Drivers → LED Arrays

# 3.  Hardware Components

## 3.1  Bill of Materials

| Component | Model / Spec | Qty | Role |
| --- | --- | --- | --- |
| ESP32 Development Board | ESP32-WROOM-32 | 1 | Main controller + web server |
| ENC28J60 Ethernet Module | SPI, 3.3V logic | 1 | Wired network interface |
| Arduino Nano | ATmega328P, 5V | 1 | Relay & LED driver control |
| 8-Channel Relay Module | 5V coil, 10A/250VAC | 1 | Power switching |
| High-Power LED Array (Red) | 354 LEDs total | 1 | STOP signal |
| High-Power LED Array (Green) | 354 LEDs total | 1 | GO signal |
| LED Driver Board | Constant current | 2 | Drive LED arrays |
| 3.3V / 5V Power Supply | VCC conversion | 1 | Level shifting |
| Ethernet Cable (CAT5e/6) | RJ45 | 1 | Network connection |
| Mounting Poles / Housing | Weatherproof | 1 | Outdoor installation |

# 4.  Pin Connections & Wiring

## 4.1  ESP32 ↔ ENC28J60 (SPI Interface)

| ENC28J60 Pin | ESP32 GPIO | Notes |
| --- | --- | --- |
| Vcc (5V) | Vin → 3.3V converted | Voltage converted to 3.3V for ESP32 logic |
| GND | GND | Common ground |
| SCK | GPIO18 | SPI Clock |
| SO (MISO) | GPIO19 | Master In Slave Out |
| SI (MOSI) | GPIO23 | Master Out Slave In |
| CS | GPIO05 | Chip Select (active LOW) |

## 4.2  ESP32 GPIO → Relay Module (LN1–LN8)

| Relay Channel | ESP32 GPIO | Function |
| --- | --- | --- |
| LN1 | GPIO14 | Relay Channel 1 |
| LN2 | GPIO27 | Relay Channel 2 |
| LN3 | GPIO26 | Relay Channel 3 |
| LN4 | GPIO15 | Relay Channel 4 |
| LN5 | GPIO32 | Relay Channel 5 |
| LN6 | GPIO33 | Relay Channel 6 |
| LN7 | GPIO25 | Relay Channel 7 |
| LN8 | GPIO21 | Relay Channel 8 |

## 4.3  Relay Module Power

| Relay Module Pin | Source | Notes |
| --- | --- | --- |
| VCC (5V) | 5V Supply | Powers relay coils |
| GND | Common GND | Common ground reference |

# 5.  Web Control Interface

## 5.1  URL Endpoint Structure

The ESP32 hosts an HTTP server accessible over the local network. Commands are issued via GET requests to specific URL paths:

| URL Endpoint | Action | LED State |
| --- | --- | --- |
| http://<ip>/stop | 🔴 STOP | Red LEDs ON — Green LEDs OFF |
| http://<ip>/go | 🟢 GO | Green LEDs ON — Red LEDs OFF |
| http://<ip>/status | 📊 STATUS | Returns current state as JSON |

# 6.  Build & Assembly Guide

Step 1 — Assemble Relay & Nano Board Mount the Arduino Nano on a DIN rail bracket or prototype board alongside the 8-channel relay module. Connect the Nano's digital output pins D2–D9 to IN1–IN8 of the relay module. Ensure the relay module has its own 5V power rail separate from the Nano's power pin to avoid current overload.

Step 2 — Wire LED Drivers to Relay Outputs Connect the constant-current LED driver boards to the relay NO (Normally Open) contacts. Wire the Red LED array driver to relay channels LN1–LN4 and the Green LED array driver to LN5–LN8. Connect the LED driver input power to a suitable DC supply rated for your LED arrays.

Step 3 — Connect ESP32 + ENC28J60 Mount the ENC28J60 module on the same carrier board as the ESP32. Wire SPI connections (SCK → GPIO18, MISO → GPIO19, MOSI → GPIO23, CS → GPIO05). Run a 5V line through a voltage converter to supply 3.3V to the ENC28J60 logic. Connect the ESP32 GPIO relay control lines (GPIO14–GPIO21) to the Arduino Nano's input control bus or directly to relay IN pins.

Step 4 — Flash Firmware Upload the ESP32 sketch via Arduino IDE or PlatformIO. Configure the static IP address, subnet mask, and gateway in the code. Flash the Arduino Nano sketch for relay control logic. Test URL endpoints on the local network before physical deployment.

Step 5 — Mount LED Arrays on Poles Install the high-power red and green LED arrays on highway mounting poles at a height visible to approaching vehicles (typically 5–7 m). Ensure weatherproof enclosures (IP65 or above). Route cabling back to the control box containing the Nano, ESP32, relay module, and power supplies.

Step 6 — Deploy & Test at Toll Booth Connect the system to the toll booth's local network via the RJ45 Ethernet port. Perform end-to-end test: issue /stop and /go URL commands from the operator terminal and verify LED array switching. Configure firewall rules so only authorized operator terminals can reach the device IP.

# 7.  Importance & Smart City Applications

This system addresses a critical pain point in highway toll infrastructure by replacing manual operator signaling with a reliable, networked control mechanism. Key benefits include:

| Operational Benefits - Eliminates human signaling errors at toll gates - Faster vehicle throughput with instant light switching - Remote operation reduces booth staffing requirements - Audit trail possible via server-side request logging - Scalable to multiple lanes from a single control panel | Smart City Integration - REST API enables integration with toll management software - MQTT extension possible for centralized city traffic dashboards - ANPR camera integration for automatic payment verification - Over-the-Air (OTA) firmware updates via ESP32 Wi-Fi fallback - Data analytics on peak hours, wait times, and throughput rates |
| --- | --- |

# 8.  Safety Notes & Best Practices

⚠️ High Voltage Warning The LED driver boards and relay output circuits may operate at mains voltage levels. Ensure all high-voltage wiring is performed by a qualified electrician. Use properly rated enclosures and observe local electrical codes.

🔒 Network Security Restrict access to the ESP32 web server by IP allowlist or place the device on a dedicated VLAN. Avoid exposing the device directly to the public internet without a firewall. Consider implementing basic HTTP authentication for the URL endpoints.

🌧️ Weatherproofing All outdoor-mounted LED arrays, junction boxes, and cable entry points must be rated IP65 or higher. Apply UV-resistant conduit for all outdoor cable runs. Inspect seals and gaskets every 6 months in high-humidity environments.

# 9.  References & Acknowledgements

- ESP32 Official Documentation — espressif.com/en/products/socs/esp32

- ENC28J60 Ethernet Controller Datasheet — Microchip Technology

- Arduino Nano Reference — docs.arduino.cc/hardware/nano

- UIPEthernet Library for ENC28J60 — github.com/UIPEthernet/UIPEthernet

- Arduino 8-Channel Relay Module Reference — Generic Module Docs

IoT Toll Traffic Light System Designed for smart, efficient, and safe highway toll management.