GENXITECH SOLUTIONS Computer Vision & AI Labeling Expertise 🎯 Drone Landing Zone (LZ) Detection Dataset Dataset Creation, Annotation & Object Detection Report 259 Labeled Images; 6 Shape Classes; 0.8–1.0 Confidence. Prepared by: Amir Rehman Lead Engineer — GenXitech Solutions Airworks Society · National University of Sciences & Technology (NUST)

# 1.  Project Overview

This project demonstrates practical expertise in dataset annotation and object detection model preparation using the Roboflow platform. The core objective was to train a computer vision model capable of detecting drone landing zones (LZ) marked with pentagon shapes, while correctly ignoring a set of visually similar false decoy shapes — triangles, rectangles, hexagons, heptagons, and octagons.

🚁 Mission Goal Enable an autonomous drone to identify its pentagon-shaped Landing Zone (LZ) during flight and safely initiate a landing sequence, while reliably ignoring all decoy shapes placed in the same environment.

This work was developed under the Airworks Society at the National University of Sciences & Technology (NUST), as part of an autonomous drone vision system.

# 2.  Dataset Annotation Process

Images were uploaded and annotated using Roboflow's labeling tools. The polygon annotation tool was used to precisely label the irregular boundaries of each shape, ensuring accurate class boundaries for model training.

## 2.1  Annotation Methodology

- All 259 images were uploaded to the Roboflow project workspace.

- The polygon annotation tool was selected to trace exact shape boundaries — critical for distinguishing visually similar polygons.

- Each annotation was carefully labeled with the correct class.

- Every image was inspected post-annotation to ensure accuracy and consistency across the full dataset.

- Ambiguous or unclear images were re-labeled or removed.

## 2.2  Dataset Statistics

| Property | Value | Notes |
| --- | --- | --- |
| Total Images | 259 | Fully annotated with polygon labels |
| Annotation Type | Polygon | Precise shape boundary tracing |
| Annotation Tool | Roboflow | Cloud-based labeling platform |
| Export Format | YOLOv8 / PyTorch | Ready for model training |
| Number of Classes | 6 | LZ + 5 decoy shapes |
| Model Confidence | 0.8 – 1.0 | Across all classes post-training |

# 3.  Classes Annotated

Six distinct shape classes were defined for the dataset. The LZ class (pentagon) is the primary detection target; all other classes are decoy shapes that the model must learn to ignore.

| Class Label | Shape | Sides | Role |
| --- | --- | --- | --- |
| LZ | Pentagon | 5 sides | 🎯 Landing Zone — Primary Target |
| Triangle | Triangle | 3 sides | ⚠️ Decoy — Must Ignore |
| Rectangle | Rectangle | 4 sides | ⚠️ Decoy — Must Ignore |
| Hexagon | Hexagon | 6 sides | ⚠️ Decoy — Must Ignore |
| Heptagon | Heptagon | 7 sides | ⚠️ Decoy — Must Ignore |
| Octagon | Octagon | 8 sides | ⚠️ Decoy — Must Ignore |

⚡ Annotation Challenge Pentagon (5 sides), Hexagon (6 sides), and Heptagon (7 sides) are visually very similar, especially at drone altitude. Precise polygon annotation was essential to give the model sufficient detail to discriminate between these near-identical shapes.

# 4.  Model Training & Detection Results

After the annotation phase, the dataset was exported from Roboflow in YOLOv8-compatible format and used to train an object detection model in PyTorch. The model was trained to detect all six shape classes simultaneously, with the LZ (pentagon) class as the primary mission-critical target.

## 4.1  Training Pipeline

- Dataset exported from Roboflow (YOLOv8 / PyTorch format).

- Standard augmentation pipeline applied: horizontal flip, rotation, brightness and contrast variation.

- Model initialized from a pretrained YOLOv8 base checkpoint.

- Training run for 100 epochs at 640×640 input resolution.

- Model validated against a held-out test split after each epoch.

- Confidence threshold tuned specifically for the LZ class to minimize missed detections.

## 4.2  Detection Performance

| Class | Confidence Range | Notes |
| --- | --- | --- |
| LZ (Pentagon) | 0.88 – 1.00 | Primary target — highest precision maintained |
| Triangle | 0.82 – 0.97 | Clean discrimination from pentagon |
| Rectangle | 0.91 – 1.00 | Easiest class — distinct aspect ratio |
| Hexagon | 0.80 – 0.95 | Closest decoy to LZ — still reliable |
| Heptagon | 0.81 – 0.94 | Reliably separated from pentagon |
| Octagon | 0.85 – 0.98 | Strong detection performance |

# 5.  Outcome & Impact

The trained model was integrated into an autonomous drone navigation system developed under the Airworks Society at NUST. The system performed reliably in real-world flight tests involving multiple shapes placed on the ground simultaneously.

| ✅ Mission Outcomes - Drone correctly identifies pentagon LZ during flight - All 5 decoy shapes reliably ignored - Reduced false detections in complex environments - Improved landing accuracy during autonomous approach - Enhanced safety — no unintended landings on decoys | 🚀 Future Expansion - MQTT integration for centralized drone fleet management - Multi-LZ support for parallel runway / landing pad arrays - Night-vision dataset variant for low-light operations - Edge deployment on onboard NVIDIA Jetson module - ANPR-style integration with drone ID verification |
| --- | --- |

# 6.  Skills Demonstrated

| Skill Area | Tools & Frameworks |
| --- | --- |
| Image Annotation & Labeling | Roboflow Polygon Annotation Tool |
| Dataset Management | Roboflow (versioning, augmentation, export) |
| Object Detection | YOLOv8, PyTorch |
| Image Processing & Visualization | OpenCV, Matplotlib |
| Autonomous Drone Vision | Custom navigation system integration |
| Model Evaluation | Confidence scoring, precision/recall analysis |

GENXITECH SOLUTIONS Computer Vision & AI Labeling Expertise Autonomous Drone Vision · Roboflow · YOLOv8 · PyTorch · OpenCV